Design & Implementation of a

Campus Area Network (CAN)

CCNA Networking Project Report

Network Name:

UNIVERSITY CAMPUS AREA NETWORK

(UCAN)

Simulation Platform: Cisco Packet Tracer 8.2.2

Report Date: May 2026

# Table of Contents

Table of Contents	2

1. Executive Summary	3

2. Network Naming	3

3. Project Objectives	3

4. Network Topology Overview	3

4.1 Site Summary	3

4.2 Device Inventory	4

5. VLAN and IP Addressing Design	5

5.1 Main Campus VLANs	5

5.2 Branch Campus VLANs	5

5.3 WAN and Server Addressing	5

6. Configuration Highlights	6

6.1 Inter-VLAN Routing (Router-on-a-Stick)	6

6.2 DHCP Services	6

6.3 Access Switch Configuration	6

6.4 Dynamic Routing – RIP v2	6

7. Network Services	6

8. Security & Segmentation Notes	6

9. Observations & Recommendations	7

10. Conclusion	7

# 1. Executive Summary

This report documents the design and configuration of the University Campus Area Network (UCAN), a multi-site enterprise network built and simulated in Cisco Packet Tracer 8.2.2. The network connects a Main Campus and a Branch Campus over a serial WAN, while a third router models the external Cloud / data-centre block that hosts shared servers.

The Main Campus uses a router-on-a-stick design across eight departmental VLANs, with a dedicated DHCP pool per department. The Branch Campus serves staff and student-lab users through two additional VLANs. All three routers exchange routes dynamically using RIP version 2, providing full reachability between campuses and the server block.

In total the topology contains 39 devices: 3 routers, 2 multilayer switches, 10 access switches, 8 PCs, 9 printers, and 3 servers (Web, FTP, and Email).

# 2. Network Naming

The campus area network has been named:

University Campus Area Network — UCAN

Rationale for the name:

- “University” reflects the deployment context: the VLANs map to academic and administrative departments such as Administration, Finance, HR, IT, and Student Labs.

- “Campus Area Network (CAN)” is the correct classification: the topology spans multiple buildings/departments on a Main Campus plus a geographically separate Branch Campus, which is the textbook definition of a CAN.

- The short form UCAN is easy to use in device hostnames, documentation, and diagrams.

# 3. Project Objectives

- Segment each university department into its own broadcast domain using VLANs for security and traffic isolation.

- Provide automatic IP addressing to all end devices through DHCP.

- Enable inter-VLAN routing so departments can communicate through a central router.

- Interconnect the Main Campus, Branch Campus, and Cloud/server block over a WAN.

- Use a dynamic routing protocol (RIPv2) so all sites learn each other’s networks automatically.

- Host shared services (Web, FTP, Email) reachable from across the campus.

# 4. Network Topology Overview

UCAN is organised into three routed sites connected by serial links:

## 4.1 Site Summary

| Site | Router | Role |
| --- | --- | --- |
| Main Campus | Main Campus Router (Cisco 2911) | Router-on-a-stick for 8 departmental VLANs; central DHCP server |
| Branch Campus | Branch Campus Router (Cisco 2911) | Serves Staff and Student-Lab VLANs; local DHCP |
| Cloud / Data Centre | Cloud Router (Cisco 2911) | Hosts the shared server block (Web, FTP, Email) |

## 4.2 Device Inventory

| Category | Model | Qty |
| --- | --- | --- |
| Router | Cisco 2911 | 3 |
| Multilayer (L3) Switch | Cisco 3650-24PS | 2 |
| Access Switch | Cisco 2960-24TT | 10 |
| PC | PC-PT | 8 |
| Printer | Printer-PT | 9 |
| Server | Server-PT (Web / FTP / Email) | 3 |
| Total devices | — | 39 |

# 5. VLAN and IP Addressing Design

Each department is mapped to a dedicated VLAN, IP subnet, and DHCP pool. All subnets use a /24 (255.255.255.0) mask, and the router subinterface acts as the default gateway for each VLAN.

## 5.1 Main Campus VLANs

| Department | VLAN | Network | Gateway | DHCP Pool |
| --- | --- | --- | --- | --- |
| Administration | 10 | 192.168.1.0/24 | 192.168.1.1 | admin-pool |
| Human Resources | 20 | 192.168.2.0/24 | 192.168.2.1 | HR-pool |
| Finance | 30 | 192.168.3.0/24 | 192.168.3.1 | Finance-pool |
| Business | 40 | 192.168.4.0/24 | 192.168.4.1 | bus-pool |
| E&C | 50 | 192.168.5.0/24 | 192.168.5.1 | ec-pool |
| A&D | 60 | 192.168.6.0/24 | 192.168.6.1 | A&D-pool |
| Student Lab | 70 | 192.168.7.0/24 | 192.168.7.1 | student-lab-pool |
| IT | 80 | 192.168.8.0/24 | 192.168.8.1 | it-pool |

## 5.2 Branch Campus VLANs

| Department | VLAN | Network | Gateway | DHCP Pool |
| --- | --- | --- | --- | --- |
| Staff | 90 | 192.168.9.0/24 | 192.168.9.1 | staf-pool |
| Student Lab (Branch) | 100 | 192.168.10.0/24 | 192.168.10.1 | studlb-pool |

## 5.3 WAN and Server Addressing

| Link / Segment | Address | Mask |
| --- | --- | --- |
| Main ↔ Branch (Serial0/1/1 ↔ Serial0/2/0) | 10.10.10.0/30 | 255.255.255.252 |
| Main ↔ Cloud (Serial0/1/0 ↔ Serial0/1/0) | 10.10.10.4/30 | 255.255.255.252 |
| Cloud Router GigabitEthernet0/0 (server LAN) | 20.0.0.1/30 | 255.255.255.252 |
| Email Server | 20.0.0.2 | 255.255.255.252 |

# 6. Configuration Highlights

## 6.1 Inter-VLAN Routing (Router-on-a-Stick)

The Main Campus Router uses one physical GigabitEthernet0/0 interface divided into eight 802.1Q subinterfaces — one per VLAN — each carrying the gateway IP for its department. Example:

interface GigabitEthernet0/0.10

encapsulation dot1Q 10

ip address 192.168.1.1 255.255.255.0

## 6.2 DHCP Services

Each VLAN has its own DHCP pool defined on the router, supplying the network, default-router, and dns-server for that department. End devices (PCs and printers) obtain addresses automatically. Example pool:

ip dhcp pool admin-pool

network 192.168.1.0 255.255.255.0

default-router 192.168.1.1

## 6.3 Access Switch Configuration

Each departmental access switch (Cisco 2960-24TT) has all its access ports assigned to that department’s VLAN, with the uplink configured as an 802.1Q trunk to carry tagged traffic to the router/L3 switch. For example, the ADMIN switch places ports Fa0/1–Fa0/x in VLAN 10, while HR uses VLAN 20, Finance VLAN 30, and so on.

## 6.4 Dynamic Routing – RIP v2

All three routers run RIP version 2 so each site automatically learns the others’ networks. The Main Campus Router advertises all eight departmental subnets plus the 10.0.0.0 WAN; the Branch and Cloud routers advertise their local networks. RIPv2 was chosen because it is classless, supports VLSM, and is simple to configure for a campus of this size.

router rip

version 2

network 10.0.0.0  ...  network 192.168.8.0

# 7. Network Services

The Cloud / data-centre block hosts three shared servers that are reachable from all campus VLANs through RIP-advertised routes:

| Server | Service | Purpose |
| --- | --- | --- |
| WEB SERVER | HTTP / HTTPS | Hosts internal university web portal and pages |
| FTP | File Transfer | Central file storage and software distribution |
| EMAIL SERVER | SMTP / POP3 | Campus email; addressed at 20.0.0.2 |

# 8. Security & Segmentation Notes

- VLAN segmentation keeps each department in its own broadcast domain, limiting the scope of broadcasts and isolating sensitive groups such as Finance and HR.

- An access list (access-group 10) is applied outbound on the Branch Campus Router serial interface, demonstrating basic traffic filtering between sites.

- Inter-VLAN traffic is forced through the router, which is the natural point to add further ACLs or firewall policy.

# 9. Observations & Recommendations

The network is functional and demonstrates the core CCNA topics well. The following improvements would strengthen it for a production-grade deployment:

- Standardise hostnames: several devices still show the default “Router” / “Switch” hostname in their running-config. Renaming them to match the topology labels (e.g. MAIN-RTR, BRANCH-RTR, ADMIN-SW) improves manageability.

- Correct the Branch Router RIP statements: the branch advertises 198.168.9.0/198.168.10.0, but the interfaces use 192.168.9.0/192.168.10.0. Aligning these ensures the branch LANs are advertised correctly.

- Enable password protection: console, VTY, and enable secrets are currently open. Add authentication and 'service password-encryption' for security.

- Consider OSPF instead of RIPv2 for faster convergence and better scalability if the campus grows.

- Use the multilayer (3650) switches for inter-VLAN routing (SVIs) to offload the router and improve performance.

# 10. Conclusion

The University Campus Area Network (UCAN) successfully demonstrates a complete small-enterprise campus design: VLAN segmentation per department, router-on-a-stick inter-VLAN routing, automatic DHCP addressing, a serial WAN linking two campuses and a server block, dynamic routing with RIPv2, and shared Web/FTP/Email services. With the minor naming, addressing, and security refinements noted above, the design would be ready for a production campus environment.